Uganda Refugee Settlement Road Digitization Protocol:

Introduction:

The United Nations High Commissioner for Refugees (UNHCR) estimates that in 2024 122.6M people were forcibly displaced worldwide (Global-Report-2024, n.d.). As displacement increases, the populations of refugee settlements are expanding rapidly, and new camps continue to be established (Van Den Hoek & Friedrich, 2021). Although camps are often thought of as temporary solutions constructed utilizing cheap building materials and basic design configurations (Gupta et al., n.d.), many refugee camps have persisted for decades, transitioning into more permanent settlements (Alqub & Matar, 2025). Two thirds of refugees live in protracted refugee scenarios, and the average stay in a refugee camp in more than 10 years (Peters & Van Den Hoek, 2021).

Despite the growing number of people living in refugee settlements, they are among the most under-documented human landscapes globally. Refugee settlements are poorly accounted for in existing global settlement datasets, which often fail to detect them due to their rapid evolution, and unique material conditions (Van Den Hoek & Friedrich, 2021).

As of 2021, Uganda was host to 1.4 million refugees, which represents ~3% of its total population (Van Den Hoek & Friedrich, 2021). In 2020 this was the largest refugee population in Africa, and the second largest in the world (Friedrich & Van Den Hoek, 2020). The refugee population increased by ten-fold between 2012 and 2017 as a result of humanitarian crises in Sudan and Democratic Republic of Congo. 92% of refugees in the country reside inside UNHCR refugee settlements, and the remaining 8% are in Kampala.  In 1998 Uganda codified its Self Reliance Strategy (SRS), a refugee policy that allocates land and grants refugees access to food, aid, health, and education. The policy also grants refugees the ability to freely travel outside the borders of refugee settlements for work (Bjørkhaug, 2020). As a result, Uganda is unique among refugee hosting countries in how refugee populations are integrated into surrounding host country communities (Bohnet & Schmitz-Pranghe, 2019).

Among the less documented components of humanitarian landscapes are roads and footpaths. These infrastructure features mediate many aspects of daily life, including access to water, markets, and agricultural fields. Yet, in many cases, maps of roads are incomplete and inconsistently updated. Current maps rely heavily on manual digitization. Planners and researchers trace roads from very high–resolution (VHR) imagery using platforms such as QGIS or OpenStreetMap (OSM) (Haklay & Weber, 2008), or they use GPS trackers to map roadways in the field.

Objectives:

The primary goal of this project is to create a comprehensive, consistent, and up-to-date dataset representing roads and footpaths across refugee settlements in Uganda. To achieve this goal, we will rely on a standardized digitization protocol that ensures features mapped by different participants can be combined into a single, consistent data product. The resulting data product will support several vital downstream analysis tasks, such as modelling road deprivation, estimating commute times (and associated risk), and training and benchmarking deep-learning based road extraction tools. Adherence to the digitization protocol not only ensures consistent results, but also helps us assess the usefulness of such an approach for similar humanitarian mapping tasks in other regions and contexts.

Overview:

As a project participant, your role is to use a Geographic Information System (GIS) to manually digitize transportation features within an assigned portion of the study area. The study area has been divided into a regular grid, and each participant is assigned a randomly selected set of grid cells (hereafter referred to as your assignment). Participants will work one grid cell at a time, tracing all visible transportation features within each grid cell at a fixed map scale and recording a small set of standardized attributes for each feature. A grid cell should be considered complete only when all visible transportation features within its boundaries have been digitized. Each participant is responsible for fully completing all grid cells in their assignment.

A small subset of grid cells will be digitized by more than one participant. These redundant grid cells will be used to assess consistency across digitizers. The participants should treat redundant grid cells no differently than any other grid cell; they exist to help project organizers assess the effectiveness of the digitization protocol and establish baseline levels of disagreement in ground truth data.

The subsequent sections describe specific parameters of the digitization protocol and step by step instructions for operating the GIS software. Participants should carefully review the contents of these sections before beginning their digitization tasks.

Study Area:

The study area consists of refugee settlements across Uganda. Settlement extents are defined using boundaries provided by the Humanitarian OpenStreetMap Team (HOT OSM). These polygons represent the best available delineation of refugee settlement footprints.

Grid Cells:

We generated a regular mesh of square cells to cover all refugee settlements defined by the HOT OSM dataset, extending our mesh dataset 1 km beyond settlement boundaries. Grid cells measure 1 km x 1 km, forming a continuous mesh that covers settlement interiors and their direct surroundings. Each grid cell represents a unit of work. Participants should fully digitize all transportation features within the boundaries of one grid cell before moving on to the next grid cell.

Grid cells are randomly assigned to participants using a reproducible randomization procedure implemented in Python using the Geopandas and random libraries. According to the assignment process:

- Every grid cell is assigned to at least one participant.

- Approximately 5% of grid cells are intentionally assigned to multiple digitizers to introduce redundancy.

Each participant will receive a GeoJSON containing only the grid cells assigned to them, which defines the full extent of their digitization task. Each cell will have the following attributes:

OID: a unique object identifier

poly_id: a numerical identifier corresponding to a refugee settlement, grid cells that fall outside of settlement boundaries have a poly_id of -9999

redundant: a Boolean value which specifies whether the cell is shared between multiple participants

References:

Alqub, H., & Matar, O. (2025). Refugees’ home-making practices as assemblages: Material & symbolic features of housing settlements in the camp. World Development Perspectives, 39, 100714. https://doi.org/10.1016/j.wdp.2025.100714

Bjørkhaug, I. (2020). Revisiting the Refugee–Host Relationship in Nakivale Refugee Settlement: A Dialogue with the Oxford Refugee Studies Centre. Journal on Migration and Human Security, 8(3), 266–281. https://doi.org/10.1177/2331502420948465

Bohnet, H., & Schmitz-Pranghe, C. (2019). Uganda: A role model for refugee integration?

Chu, L., Oloo, F., Sudmanns, M., Tiede, D., Hölbling, D., Blaschke, T., & Teleoaca, I. (2020). Monitoring long-term shoreline dynamics and human activities in the Hangzhou Bay, China, combining daytime and nighttime EO data. Big Earth Data, 4(3), 242–264. https://doi.org/10.1080/20964471.2020.1740491

Friedrich, H. K., & Van Den Hoek, J. (2020). Breaking ground: Automated disturbance detection with Landsat time series captures rapid refugee settlement establishment and growth in North Uganda. Computers, Environment and Urban Systems, 82, 101499. https://doi.org/10.1016/j.compenvurbsys.2020.101499

Global-report-2024. (n.d.).

Gupta, A., Ortiz, A., Fobi, S., Kebut, D., Iyer, S., Dodhia, R., & Lavista-Ferres, J. (n.d.). Mapping Refugee Camps with AI: A Benchmark Dataset and Baseline Models for Humanitarian Applications.

Haklay, M., & Weber, P. (2008). OpenStreetMap: User-Generated Street Maps. IEEE Pervasive Computing, 7(4), 12–18. https://doi.org/10.1109/MPRV.2008.80

Peters, L. E. R., & Van Den Hoek, J. (2021). Charting a justice-based approach to planned climate relocation for the world’s refugees. In I. J. Ajibade & A. R. Siders, Global Views on Climate Relocation and Social Justice (1st ed., pp. 48–61). Routledge. https://doi.org/10.4324/9781003141457-4

Van Den Hoek, J., & Friedrich, H. K. (2021). Satellite-Based Human Settlement Datasets Inadequately Detect Refugee Settlements: A Critical Assessment at Thirty Refugee Settlements in Uganda. Remote Sensing, 13(18), 3574. https://doi.org/10.3390/rs13183574